8-Channel Ultrasound Receive Beamformer

Fixed-Point Delay-and-Sum (DAS) RTL for AMD/Xilinx Arty A7

This document describes a synthesizable-oriented Verilog starting point for an eight-channel ultrasound receive delay-and-sum beamformer. It uses signed fixed-point sample values, integer-sample delays, and a widened accumulator. Verify the RTL in simulation and review Vivado synthesis/timing reports before hardware use.

# 1. Architecture

Eight synchronous RF sample streams enter the module. Each channel has a sample-history buffer. A programmable delay selects either the current sample (delay 0) or a prior sample from the history. The eight selected samples are sign-extended and summed to produce one beamformed output sample when sample_valid is asserted.

Input: eight packed, signed 16-bit samples (samples_in[15:0] is channel 0).

Delay control: eight packed 5-bit unsigned values, one per channel, nominally 0–31 samples.

Output: signed 19-bit sum and a one-cycle beam_valid pulse.

Reset: clears the history and output registers.

# 2. Fixed-point representation

The RTL treats each input as a signed 16-bit two's-complement integer. If the ADC data is normalized to Q1.15, each sample has 15 fractional bits. The addition keeps the same binary point; only the integer range grows. A sum of eight signed 16-bit values fits in 19 signed bits.

| Signal | Width | Interpretation | Notes |
| --- | --- | --- | --- |
| Channel sample | 16 bits | Q1.15 if normalized | Signed two's complement |
| Two-channel sum | 17 bits | Q2.15 | Range grows by one bit |
| Four-channel sum | 18 bits | Q3.15 | Range grows by another bit |
| Eight-channel sum | 19 bits | Q4.15 | No fractional shift during addition |

# 3. Delay configuration

Each channel uses a 5-bit delay field. The packed fields are assigned from least-significant bits upward as follows:

CH0: delay_cfg[4:0]

CH1: delay_cfg[9:5]

CH2: delay_cfg[14:10]

CH3: delay_cfg[19:15]

CH4: delay_cfg[24:20]

CH5: delay_cfg[29:25]

CH6: delay_cfg[34:30]

CH7: delay_cfg[39:35]

A value of zero selects the current input sample. A value d greater than zero selects the sample stored d sample-valid events earlier. The delay values in a basic test are illustrative, not calibrated focusing delays. Actual ultrasound delays depend on element geometry, sound speed, sample rate, and focal location.

# 4. Verilog HDL source

Save the following code as ultrasound_das_8ch.v.

module ultrasound_das_8ch #(

parameter DATA_W    = 16,

parameter MAX_DELAY = 31,

parameter ACC_W     = 19

)(

input wire clk,

input wire rst,

input wire sample_valid,

// Eight packed signed 16-bit RF samples

input wire [127:0] samples_in,

// Eight unsigned 5-bit delay values, 0 to 31

input wire [39:0] delay_cfg,

output reg signed [ACC_W-1:0] beam_out,

output reg beam_valid

);

reg signed [DATA_W-1:0]

hist [0:7][0:MAX_DELAY-1];

reg signed [ACC_W-1:0] ch_ext;

reg signed [ACC_W-1:0] sum_accum;

reg signed [DATA_W-1:0] tap_sample;

reg [4:0] delay_value;

integer ch, k;

always @(posedge clk) begin

if (rst) begin

beam_out   <= 0;

beam_valid <= 1'b0;

for (ch = 0; ch < 8; ch = ch + 1)

for (k = 0; k < MAX_DELAY; k = k + 1)

hist[ch][k] <= 0;

end

else begin

beam_valid <= 1'b0;

if (sample_valid) begin

sum_accum = 0;

for (ch = 0; ch < 8; ch = ch + 1) begin

delay_value = delay_cfg[ch*5 +: 5];

if (delay_value == 0)

tap_sample =

$signed(samples_in[ch*16 +: 16]);

else

tap_sample =

hist[ch][delay_value - 1'b1];

// Sign extension: 16-bit to 19-bit

ch_ext = {{3{tap_sample[15]}}, tap_sample};

sum_accum = sum_accum + ch_ext;

end

beam_out   <= sum_accum;

beam_valid <= 1'b1;

// Update each channel's sample history

for (ch = 0; ch < 8; ch = ch + 1) begin

for (k = MAX_DELAY-1; k > 0; k = k - 1)

hist[ch][k] <= hist[ch][k-1];

hist[ch][0] <=

$signed(samples_in[ch*16 +: 16]);

end

end

end

end

endmodule

# 5. Vivado synthesis workflow

Create a new RTL project in Vivado.

Select the exact FPGA part fitted to the board: Arty A7-35T or Arty A7-100T. Confirm the part number printed in the board documentation.

Add ultrasound_das_8ch.v as a Verilog design source and set ultrasound_das_8ch as the top module.

Create a testbench and simulate reset, sample_valid, positive and negative RF values, and different channel delay settings.

Run synthesis and inspect inferred logic, resource utilization, and warnings.

Add the correct Arty A7 master XDC clock constraint for the 100 MHz oscillator, then run implementation and inspect timing reports.

# 6. Important implementation notes

This is an integer-delay DAS prototype. It does not include fractional-delay interpolation, apodization, dynamic receive-focus calculation, envelope detection, or scan conversion.

The sample history uses 8 × 31 × 16 = 3,968 bits of storage, plus selection logic. The RTL's dynamic tap selection may infer multiplexers rather than block RAM.

The accumulation is written as a loop in a clocked process. Although synthesizable in common Vivado flows, it can create a long combinational path. If timing fails, use a balanced adder tree and/or pipeline registers, preserving the sample alignment and valid latency.

The delay fields are 5 bits wide, so they encode 0–31. The history array stores 31 prior samples; check the tool's array-index and delay-bound warnings in simulation and synthesis. A production implementation should explicitly handle out-of-range delay values.

The Arty A7 does not provide eight dedicated high-speed ultrasound ADC channels. A suitable external acquisition interface or replay/test-data source is needed.

This RTL is a research/education starting point, not a validated diagnostic imaging system.

# 7. Basic functional checks

All channel samples zero: beam_out should be zero.

All delays zero and all eight samples equal +100: beam_out should be +800.

All delays zero and all eight samples equal −100: beam_out should be −800.

Only CH0 has a nonzero sample and its delay is zero: output should follow CH0 with beam_valid asserted on valid input cycles.

Set a channel delay to a nonzero value and verify against a sample-by-sample reference model.

# 8. Mathematical model

The delay-and-sum operation is:

y[n] = Σ(i=0 to 7) xᵢ[n − dᵢ]

where xᵢ is the RF sample stream for channel i and dᵢ is that channel's integer delay. Summation does not require a binary-point shift. If an average is desired, divide the sum by eight using an arithmetic right shift by three, while considering signed rounding behavior.