8-Channel Ultrasound Receive Beamformer

Floating-Point Delay-and-Sum (DAS) Design for AMD/Xilinx Arty A7

Important synthesis note: IEEE-754 floating-point arithmetic is not implemented by ordinary Verilog '+' on raw 32-bit bit vectors. The source below is an interface/architecture template, and its placeholder fp32_add function returns zero. It is intentionally not a working beamformer until the placeholder is replaced with a real floating-point adder IP implementation.

# 1. Architecture

The design stores IEEE-754 binary32 sample words for each of eight channels, selects a delayed sample per channel, and accumulates the eight values. Each channel sample is a 32-bit floating-point word: 1 sign bit, 8 exponent bits, and 23 fraction bits. The delay history is independent of the numeric format.

Input: eight packed 32-bit IEEE-754 binary32 values (256 bits total).

Delay control: eight packed 5-bit integer delay values, nominally 0–31.

Output: one 32-bit IEEE-754 binary32 result and beam_valid.

Reset: clears the history and output to the binary32 bit pattern for +0.0.

# 2. Floating-point representation

IEEE-754 binary32 uses one sign bit, eight exponent bits, and 23 fraction bits, with an implicit leading significand bit for normalized values. Unlike fixed-point arithmetic, the binary point varies with the exponent. Summing eight values can introduce rounding at each floating-point addition, and the result can differ slightly depending on the adder-tree order.

| Field | Bits | Meaning |
| --- | --- | --- |
| Sign | 1 | Positive or negative |
| Exponent | 8 | Biased exponent |
| Fraction | 23 | Significand fraction |

# 3. Delay configuration

The delay_cfg bus contains eight 5-bit fields, least-significant field first: CH0 is delay_cfg[4:0], CH1 is [9:5], CH2 is [14:10], CH3 is [19:15], CH4 is [24:20], CH5 is [29:25], CH6 is [34:30], and CH7 is [39:35]. A zero delay selects the current sample; a positive delay selects a previous sample from that channel's history. These are integer-sample delays, not automatically calculated ultrasound focusing delays.

# 4. RTL template

Save as ultrasound_das_8ch_fp32.v. The fp32_add function below is explicitly a placeholder and must be replaced before synthesis/use. It is included to show where floating-point addition belongs in the datapath.

module ultrasound_das_8ch_fp32 #(

parameter integer MAX_DELAY = 31

)(

input  wire         clk,

input  wire         rst,

input  wire         sample_valid,

// IEEE-754 binary32 samples:

// CH0 = samples_in[31:0], CH1 = samples_in[63:32], ...

input  wire [255:0] samples_in,

// Eight 5-bit integer-sample delays (0 to 31)

input  wire [39:0]  delay_cfg,

output reg  [31:0]  beam_out,

output reg          beam_valid

);

// Floating-point sample history, one array per channel.

// Each entry is a 32-bit IEEE-754 binary32 bit pattern.

reg [31:0] hist [0:7][0:MAX_DELAY-1];

reg [31:0] tap_sample;

reg [31:0] sum_accum;

reg [4:0]  delay_value;

integer ch, k;

// Floating-point operators below are behavioral arithmetic.

// For portable FPGA synthesis, replace + with instantiated,

// configured floating-point IP cores (e.g. Vivado Floating-Point

// Operator IP) and pipeline/control the valid signals accordingly.

always @(posedge clk) begin

if (rst) begin

beam_out   <= 32'h00000000; // +0.0 in IEEE-754

beam_valid <= 1'b0;

for (ch = 0; ch < 8; ch = ch + 1)

for (k = 0; k < MAX_DELAY; k = k + 1)

hist[ch][k] <= 32'h00000000;

end

else begin

beam_valid <= 1'b0;

if (sample_valid) begin

sum_accum = 32'h00000000; // +0.0

for (ch = 0; ch < 8; ch = ch + 1) begin

delay_value = delay_cfg[ch*5 +: 5];

if (delay_value == 0)

tap_sample = samples_in[ch*32 +: 32];

else

tap_sample = hist[ch][delay_value - 1'b1];

// IEEE-754 floating-point addition.

// Requires floating-point adder IP for FPGA synthesis.

sum_accum = fp32_add(sum_accum, tap_sample);

end

beam_out   <= sum_accum;

beam_valid <= 1'b1;

for (ch = 0; ch < 8; ch = ch + 1) begin

for (k = MAX_DELAY-1; k > 0; k = k - 1)

hist[ch][k] <= hist[ch][k-1];

hist[ch][0] <= samples_in[ch*32 +: 32];

end

end

end

end

// Placeholder function: not synthesizable as written.

// Replace this function with an actual IEEE-754 adder IP interface

// or use the fully wired IP-core implementation described below.

function [31:0] fp32_add;

input [31:0] a;

input [31:0] b;

begin

fp32_add = 32'h00000000;

end

endfunction

endmodule

# 5. Making it synthesizable in Vivado

Use the AMD/Xilinx Floating-Point Operator IP (or equivalent verified IEEE-754 IP) for addition. For a practical implementation, instantiate eight floating-point adders arranged as a balanced adder tree, or use a sequential adder with handshaking. The IP configuration determines latency and throughput, so valid signals and sample alignment must be pipelined.

Create the project for the exact Arty A7-35T or Arty A7-100T FPGA part.

In Vivado IP Catalog, configure Floating-Point Operator IP for single-precision (32-bit) addition.

Connect delayed sample words to the adder datapath. A balanced tree sums pairs, then groups of four, then the final eight-channel result.

Pipeline valid signals to match each adder's configured latency.

Run behavioral simulation, synthesis, implementation, and timing/resource checks.

For sustained one-sample-per-clock throughput, verify the adder IP's initiation interval and the available FPGA resources.

# 6. Floating-point arithmetic model

y[n] = Σ(i=0 to 7) xᵢ[n − dᵢ]

The mathematical delay-and-sum equation is unchanged. The difference is that each xᵢ and y is represented in IEEE-754 binary32 and each addition is performed by a floating-point operator. Floating-point addition is more resource-intensive than fixed-point addition and is not usually the first choice for high-throughput FPGA beamformers unless its dynamic range or algorithmic requirements justify the cost.

# 7. Limitations and verification

The included fp32_add placeholder returns zero and is not a functional IEEE-754 adder. Replace it with real floating-point IP before relying on simulation or synthesis results.

The shown clocked loop assumes zero-latency arithmetic; real floating-point IP has latency and handshake requirements. The loop must be redesigned around the chosen IP latency.

The sample-history memory and dynamic tap selection need synthesis review; they may infer registers and multiplexers.

This is an architecture template, not a validated medical imaging design.